# Architectural Blossoming of the Lotus

Exported from Holy-Writings.com on 2026-09-17 - 1 clipping

Source: Bahá'í Library Online (bahai-library.com), curated by Jonah Winters. Used by permission of the curator. Original citation: S. Naharoy, Architectural Blossoming of the Lotus, bahai-library.com.

──────────────────────────────────────────────────────────────────────

ARCHITECTURAL BLOSSOMING OF THE LOTUS

by S. Naharoy

The temples of the Baha'i Faith are well known for their     administrative building; and the restrooms block. The

architectural splendor, and the Temple constructed in        temple proper comprises a basement to accommodate the

Delhi is a continuation of this rich tradition. Before       electrical and plumbing components, and a lotus-shaped

undertaking the design of the temple, the architect, Mr.     superstructure to house the assembly area.

Fariborz Sahba, had travelled extensively in India to            All around the lotus are walkways with beautiful

study the architecture of this land and was impressed by     curved balustrades, bridges and stairs, which surround

the design of the beautiful temples, as well as by the art   the nine pools representing the floating leaves of the

and religious symbols wherein the lotus invariably           lotus. Apart from serving an obvious aesthetic function,

played an important role. He was influenced by this          the pools also help ventilate the building.

experience, and in an attempt to bring out the concept           The lotus, as seen from outside, has three sets of

of purity, simplicity and freshness of the Baha'i Faith,     leaves or petals, all of which are made out of thin

he conceived the Temple in Delhi in the form of a lotus.     concrete shells. The outermost set of nine petals, called

The temple gives the impression of a half-open lotus         the 'entrance leaves', open outwards and form the nine

flower, afloat, surrounded by its leaves. Each component     entrances all around the outer annular hall. The next set

of the temple is repeated nine times. Flint 8: Neill         of nine petals, called the 'outer leaves', point inwards.

Partnership of London were the consultants and the ECC       The entrance and outer leaves together cover the outer

Construction Group of Larsen 8: Toubro Limited were the      hall. The third set of nine petals, called the 'inner leaves',

contractors responsible for constructing the Temple.         appear to be partly closed. Only the tips open out,

The temple complex, as seen from the layout, consists   somewhat like a partly opened bud. This portion, which

of the main house of worship; the ancillary block which      rises above the rest, forms the main structure housing the

houses the reception centre, the library and the             central hall. Near the top where the leaves separate out,

nine radial beams provide the necessary lateral support.                                Geometry

Since the lotus is open at the top, a glass and steel roof

The beautiful concept of the lotus, as conceived by the

at the level of the radial beams provides protection from

architect, had to be converted into definable geometrical

rain and facilitates the entry of natural light into the

shapes such as spheres, cylinders, toroids and cones.

auditorium.

These shapes were translated into equations, which were

Below the entrance leaves and outer leaves, nine

then used as a basis for structural analysis and

massive arches rise in a ring. A row of steps through

engineering drawings. The resultant geometry was so

each arch lead into the main hall (see Fig. 1).

complex that it took the designers over two and a half

years to complete the detailed drawings of the temple.

Fig 1. Top view of entrance and outer leaves        An attempt is made below to describe this complex

geometry in simple terms (see Fig. 2).

Entrance leaves and outer leaves.

The shell surfaces on both sides of the ridge of the

entrance and outer leaves are formed out of spheres of

different radii, with their centres located at different

points inside the building. There is one set of spheres for

the entrance leaves, some of which define the inner

Fig 2. Section through entrance leaf and interior dome

(Plan and section at crown of dome also shown)

1. Entance leaf

2. Outer leaf

3. Interior dome shell

4. Arch

5. Interior dome rib

The inner leaves enclose the interior dome in a

canopy made of crisscrossing ribs and shells of intricate

pattern. When viewed from inside, each layer of ribs and

shells disappears as it rises, behind the next, lower layer

(see section on p. 29). Some of the ribs converge radially

and meet at a central hub. The radial beams emanating

from the inner leaves described earlier meet at the centre

of the building and rest on this hub. A neoprene pad is

provided between the radial beams and the top of the

interior dome to allow lateral movement caused by the

effects of temperature changes and wind.

surfaces, and others which define the outer surfaces of        of adjacent arches, thus forming an intricate pattern.

the shells. The diameters of the spheres have been fixed       Other radial ribs rise from each of these intersections and

to satisfy the structural consideration of varying shell       all meet at the centre of the dome.

thickness. Similarly, for the outer leaves, another set of

Up to a certain height, the space between the ribs is

spheres defines the inner and outer surfaces of the shells.

covered by two layers of 60mm-thick shells. The intricate

However, for the outer leaves, the shell is uniformly 133

pattern of the interior dome is illustrated in section on

mm thick towards the bottom, and increases to 255 mm

page 29.

up to the tip, beyond the glazing line.

The entrance leaf is 18.2m wide at the entrance and                                 Setting out

rises 7.8m above the podium level. The outer leaf is

The setting out of the surface geometry posed a difficult

15.4m wide and rises up to 22.5m above the podium.

task. Unlike conventional structures for which the

The inner leaves.                                              elements are defined by dimensions and levels, here the

Each corrugation of the inner leaf, compnsmg a cusp            shape, size, thickness, and other details were indicated

(ridge) and a re-entrant (valley), is made up of two           in the drawings only by levels, radii, and equations.

toroidal surfaces. A toroid is generated when a circle of      These parameters, therefore, had to be converted into a

a certain radius, 'r', is rotated around the centre of a       set of dimensions in terms of length, breadth, height,

circle of much larger radius, 'R'. A cycle tube is a typical   and thickness, easily understood by a site engineer or a

toroid. The shaded portion of the toroid is a part of the      carpentry foreman. To achieve this, a system of

inner leaf shell.                                              coordinates along x, y and z axes for every 40 degrees.

segment of the temple was worked out with the help of

The inner leaves rise to an elevation of 34.3m above

a computer. The problem was then further simplified by

the inner podium. At the lowest level each shell has a

working out from these co-ordinates levels and distances

maximum width of 14m. It is uniformly 200mm thick.

The arch.                                                                 Fig 3. Station points for setting out of arch,

entrance, outer and inner leaves

All around the central hall are nine splendid arches

placed at angular intervals of 40 degrees. The shape of

these arches is formed by a number of plane, conical and

cylindrical surfaces. The intersection of these surfaces

provides interesting contours and greatly enhances the

beauty of the arches. The nine arches bear almost the

entire load of the superstructure (see Fig. 2 and 4).

The interior dome.

Three ribs spring from the crown of each arch. While the

central one (the dome rib) rises radially towards the

central hub, the other two (the base ribs) move away

from the central rib and intersect with similar base ribs

which a carpenter or a reinforcement fitter could easily                        Fig 5. Setting out of surface

comprehend and then arrive at the surfaces and

z               ~yel

boundaries. Eighteen reference stations were established

plum~T

B

outside the building for setting out the arches, entrance,                                                      Stepped template

outer and inner leaves (see Fig. 3).                                                                bob  I

First, 18 radial lines were established from the centre

of the building (see Fig. 4). Along these lines, using

inclined and vertical distances, end points A and B for

surface (1) were established. By using a set of curved

templates, each of varying curvature, surface (1) between

y

these lines was developed. From this surface the other

surfaces of the arch were set out by using stepped

templates with respect to surface (1).

Sequences of construction

The stations shown in Fig. 3 were used to set out the     The basement and the inner podium were constructed

cusp, re-entrance and edge lines for the entrance, outer       first. Thereafter, for casting the arches and shells, the

and inner leaves. For example, to arrive at curve AB,          structure was divided into convenient parts, taking into

point A with coordinates XA, YA, ZA was defined with           consideration that when deshuttered, the portion of the

respect to O. AB was then established by a second              shells cast would be self-supporting until the remaining

theodolite and the curve AB determined by a stepped

shells were completed. The structure was divided as

template. Accurately made curved templates of required         follows:

radii were then used to develop the surface between

these boundaries (see Fig. 5).                                 Arch.

All 9 arches were cast one after the other in two lifts

Fig 4. Setting out of arch                 until the circle was completed. The deshuttering of the

soffit of each arch was taken up after the adjacent arches

had attained specified strength (see Fig. 8).

Inner leaf, radial beams and central hub.

After the completion of all the arches, the structural

steel staging for the inner leaf was erected. Three shells,

120 deg. apart, were taken up at a time and cast in two

Building                                                 lifts, one after the other, up to the radial beam level,

R                                     ensuring always that the difference in height between

the shells cast was not more than one lift (see Fig. 6).

The process was repeated until all 9 segments were cast.

\ Theodolite                                              Casting of the central hub was taken up as an

independent activity, and after all the shells were cast,

they were connected to the hub by casting the radial                outer leaves and followed by the intermediate entrance

beams. After sufficient curing, the inner leaf along with           leaf. In this manner the remaining leaves were

the radial beams were dewedged, leaving the central hub             deshuttered as and when the concrete attained strength

supported. The remaining portion of the inner leaf was              and the leaves adjacent to the shell to be deshuttered

then taken up (see Fig. 7).                                         were cast.

Fig 6. Sequence of co nstruction of entrance leaf, outer leaf                   Staging and !ormwprk

and in ner leaf

Deflection was an important consideration in the design

of the formwork. The maximum deflection was limited

to 3mm over a distance of 1m (including errors in

fabrication and erection).

The following aspects were considered in arriving at

the general arrangement of the staging supporting the

inner leaf and interior dome formwork:

a. The concreting of the shells should be taken up 3 at

a time, 120 deg. apart, so that the lateral loads on the

staging supporting the formwork were reduced as far

as possible.

b.   Construction joints were to be avoided as far as

possible so that the exposed concrete surface did not

Interior dome.                                                           show any lines other than the architectural pattern.

After de-wedging of inner leaf, the steel staging was                    For the inner leaf, construction joints were to be

modified and two folds of shells of the interior dome                    located above 24.8m level so that they did not show

taken up one after another. For each fold, three shells,                 from the floor level. All other shells were to be cast

120 deg. apart, were taken up at a time and cast one after               in a single continuous pour.

another. For each shell the boundary ribs were taken up             c.   The staging should support the radial and base ribs

first and then the shell cast in one single lift. The process            without interfering with the structural steel

was repeated until all the shells were completed.                        members. After deshuttering of inner leaf, the

structure should be able to support the formwork of

Entrance and outer leaves.

the inner layers of shells of the interior dome with

The construction of the entrance and outer leaves was

minimum modification.

taken up as a parallel activity with the casting of the

inner leaves and interior dome. Two entrance leaves and                  From the above considerations, a space frame

one intermediate outer leaf were taken up first.                    consisting of 9 radial cusp frames and 9 re-entrant

Thereafter, the outer and entrance leaves were cast                 frames, with circumferential and diagonal members

alternately, the outer leaf first and then the adjacent             closely following the profile of ribs and shells, was

entrance leaves. Deshuttering was started with a pair of            considered most suitable (see Fig. 7).

Fig 7, Inner lear and radial beams delihulterrd       deflections due to slippage of joints would be avoided and

with c~n!ral hub supported on staging

fabrication and ereclion would be comparatively easier.

The inner surfaces of all the shell s have a uniform,

bush-hammered, exposed concrete surface with

archit«tural patterns. For the inner leaves, these patterns

were formed out of radial and venical planes intersecting

the surface of the torus. For the outer and efilrdnce leaves,

and the interior dome, the patterns were formed out of

longitudes and latitudes of spheres. The form work was

designed in a manner thai limber joists support the panels

instead of the regular pauern of the structural steel

supporting members of the space frame (see Fig. 8).

Full-scale mockups of the bottom surface of each of

the shells were first made at ground level and the

archi leclUra l patterns marked on this su rface. The fra me

Various alternatives were considered for the steel        of each form panel was fabricated according to

staging. Standard pipe scaffolding was found to be             calculated dimensio ns and cross-checked with

unsuitable, considering that the slippage of members at        measurements from the mockUp. The fo rmwork pattern

joints would be uncertain and it would be difficult to          is seen in the photograph on page 70.

compute and control the deflection, particularly due to             The inner fo rmwork for every petal was fully fixed

lateral loads. Structural steel framework with boltedjoints    from bottom to lOp and aligned accurately. After the

was found to be unsatisfactory, considering that a very        form work was approved, the sheathing joints where sealed

high degree of accuracy in fabrication and erection of         with putty made out of epoxy resin and plaster of Paris.

structural work would be required to match the boll holes      and a protective coating was applied over the

at junctions of members meeting at different inclinations      plywood surface, In the case of the interior dome shells,

in all three planes. Structural steel framework with welded   the plywood sheathing was lined by fi ber-reinforced

joints was considered to be most suitable because              plastic sheets and thejoinrs sealed with epoxy resin. After

this, the locatio"n of each reinforcement bar was marked on

Ihe fomnvork along latitudes and longitudes and the bars

placed over (he markings. To avoid impressions of cold

joints on the inner surface, the casting of petals of the

inner leaf was carried out in three lifts, some of them 14m

high. To facilitate placement of concrete and si multaneous

compaction in each pour, the oUier form work was placed

one row of panels at a time, and as the level of co ncrete

rose, the next row of panels was fixed. These panels,

therefore, had to be fixed in position and aligned

Vi~w showing n~wly concreted    main archeli        accurately in the shonest possible lime.

Fig B. Formwork details of entrance leaf                             rv. The greater of dead load of concrete (or) liquid

pressure at any point corresponding to the rate of

1      placement 0.45 m/hr and minimum temperature of

10 deg. C (during winter). Concrete pressure was

calculated as per ACI publication - SPA.

Liquid pressure p = 7.2 + ([785Rl/[Tc + 17.8])

P = Lateral liquid pressure - KN/m2

R = Rate of placement - m/hr

Tc= Temperature of concrete in the forms deg. C

V. Basic wind pressure = 1000 N/m2

~    _    _ _ _ _ _ __    _   _ _ __       _   _   _ _ _ __ __ _ 1

Fig 9. Details of formwork in inner leaf

Through selected points matching with the architectural

pattern, pipe supports were taken from the inner leaf

staging. These pipes supported a structural steel grid closely                                         Detail of shaped member and back fonn

following the profile of the outer surface of the shells. The

grid supported the outer formwork against the concrete

pressure and also accommodated the working platforms at

all levels. Through-ties connecting the inner and outer

forms were provided at selected points so as to reduce the

load on the steel staging and limit the deflection of

formwork.

The longitudinal support members of the backform

had accurately aligned shaped members, such that when

the backform panels were placed in position and wedged,

the outer surface of the shell was attained without

further alignment (see Fig. 9). To ensure that the panels

fitted exactly between the shaped members and there

was no delay, the fixing of the panels for the entire shell

was carried out in advance.

2. Inner form

3. Outer form

4. Outer steel staging

Loading                                5. Working Platform

6. Concrete shell

The following loads were considered for the design of the            7.   10 mm thi ck rubber washer

B. 40 mm di ameter pipe

formwork:

9. 60 mm di ameter pvc sleeve

10. Shaped member

I.   Dead load of formwork - 750 N/m2 of surface area.               II. Back form panel

II. Self-weight of structural steel members.                         12. Longitudinal member

III. Live load 2000 N/m2 of plan area.                               13. Wedg es

For the inner leaf, various combinations of the above loads                                     Based on the above loads, a computer analysis for all

were considered for the following conditions (see Fig. 4):                                        possible combinations was carried out using SAP N

program. One cusp frame and one re-entrant frame along

Stage I Concrete from top of arch to +24.8m level

with inter-connecting bracings were considered as a unit.

Stage II Concrete from +24.8m to +38m level

Stage TIl Concrete from +38.8m to the top                                                              A computer model indicating the loads due to one of

the combinations of loading for Stage II is shown in Fig. 10.

The combination of loads considered were:                                                              Similar loading conditions were considered for the

entrance and outer leaves as also the shells of the

1. Self-weight of space frame (symmetrical)

2. Dead load of shutter                                                                           interior dome, the only difference being that all the

3. Live load + dead load of concrete Stage I (unsymmetrical)                                      shells were cast in a single pour.

4. Live load + dead load of concrete Stage I (symmetrical)

5. Live load + dead load of concrete Stage II (unsymmetrical)

6. Live load + dead load of concrete Stage II (symmetrical)                                                            Reinforcement

7. Live load + dead load of concrete Stage TIl (unsymmetrical)

8. Live load + dead load of concrete Stage TIl (symmetrical)                                      The reinforcement used in the white concrete shells as

9. Wind load for full height (unsymmetrical)                                                      well as the binding wires was entirely galvanized so as

to   prevent   the   long-term     effect   of rusting    of

reinforcement on the white concrete. Since galvanized

Fig 10. Computer diagram of nodal lo ads for inner lea f                                          reinforcement for concrete is seldom used in this

country, several tests were carried out to ensure that the

mechanical properties of reinforcement did not become

5.45        adversely affected due to galvanizing. Sandblasting was

1'1 3.61          carried out to reduce pickling time with a view to

15.83 778

/               avoiding hydrogen embrittlement. The bottom formwork

18.52

for one shell for each of the leaves was first erected and

aligned. The edge lines and surfaces of this formwork

were then used as a mockUp to decide the length and

shape of each bar in the shell. To avoid the impression of

cover blocks on the exposed surface of the shells, the

inner layer of reinforcement was held in position by

special steel spacers supported from the outer formwork.

Concrete

Forces a re in Kn

All the ribs and shells up to radial beam level are in

3.1(1"0.84

1.09          CuSp frame                                     Re-entrant frame                white concrete. To avoid crazing and shrinkage cracks, a

(    Out of plane horizontal load

mix of M 30 grade white concrete was designed

In Plane/ l ' "

horizontal load            ' " Out of plane                                                       considering that the cement content should be below 500

Vertical load

horizontal load

I "'"

Vertical load

In plane horizontal load

kg/m3 and the quantity of water reduced to a minimum.

Tests carried out on Indian cement revealed that the           constructed in in-situ concrete using formwork on both

strength and other properties varied considerably and          faces. Considering that each shell had to be cast in a

the colour did not meet the architectural requirement.         single pour, the fIxing of formwork and reinforcement,

Trial mixes also showed a higher cement requirement of         as also the placement and compaction of concrete

430-450 kg/m3. The entire quantity of white cement was         between two faces of formwork only 60 mm apart, posed

therefore imported from Korea. With the imported               serious problems. Not only was the formwork diffIcult to

cement, it was possible to produce concrete having 28          align so as to accurately produce the complex, doubly

days cube strength of 55-60 N/mm2 with a cement                curved surface and the intersections, but also the

content of 380 to 400 Kg/m3. A mix of 1: 1.44:3.36 and         closeness of the petals, one fold behind the next, caused

w/c ratio of .42 was adopted. To achieve a high                serious problems of work space for fIxing formwork,

workability, slump 1-120 mm, super plasticiser, .5 to          reinforcement and concreting.

.75% by weight of cement was used.

Specially graded dolomite aggregates were procured                           Quality assurance

from the Alwar mines near Delhi and white silica sand

Based on the sequence of construction envisaged, the

from Jaipur. The maximum temperature of concrete at

assumptions made in the design of the formwork, the

the time of placing was limited to 30 deg. C. During the

procedures developed from mockups, and the tests

summer months, when the ambient temperature was as

carried out on materials, detailed method statements and

high as 45 deg. C, the temperature of the concrete was

criteria of acceptance were established. Checking of

controlled by adding a measured quantity of ice and by

workmanship was done at each stage to produce the

the precooling of aggregates in air-cooled aggregate

required quality and accuracy and also to ensure that

storage bins. To avoid cold joints due to stoppage of

there was no deviation from the conditions of loading

work during heavy rains, as also to protect rain water

assumed in the design of the formwork. A full-fledged

entering the forms, the entire concreting area was

concrete laboratory carried out mix designs for different

covered by tarpaulins.

grades of concrete and exercised strict control on the

After removal of the outer forms, the surface of the

quality of concrete.

concrete was covered with hessian and cured for 28

days by keeping it wet continuously by a sprinkler

arrangement fIxed at the top of the shells.

Marble cladding

The outer surface of the shells, as also the inner surface of

the arches, are cladded with white marble panels fIxed to

Trials and mockups                            the concrete surface with specially designed stainless steel

The shells of the interior dome were initially 50mm thick      brackets and anchors. 10,000 sq.m. of marble was quarried

and proposed to be cast by in-situ guniting. Full-scale        from the Mount Pentilekon mines of Greece and thereafter

mockups were used to study the problems of working             sent to Italy, where each panel was cut to the required size

space and accessibility, and it was felt that due to limited   and shape to suit the geometry and architectural pattern

space available between the shells, the working                before transporting them to the site in Delhi.

conditions for guniting operations would be diffIcult. As          After waterproofIng of the top surface of each shell,

an alternative, the shells were therefore proposed to be       timber templates of the same size as the marble panels

were used to define the location of the bottom-most rows                       that the marble fixing could be carried out without any

of marble panels first. The geometry of the cusp re-entrant                    hindrance from the supports of the staging.

and edge lines was then accurately checked with respect                           It may be interesting to note that all the marble work

to these panels, and the marble pieces were fixed in                           was carried out by carpenters who learned the skill of

position from bottom towards top and cusp towards re-                          marble fixing within a few weeks, and were able to

entrants and edges. Edge holes were drilled at ground                          complete the work, to the required accuracy, two months

level for each marble panel before the panels were placed                      ahead of the scheduled completion time.

in position. Holes were drilled in the concrete to

accommodate the anchor fasteners of the stainless steel                                       Project management

brackets to suit the holes in the marble, after each panel

The complexity of the structure, and the very high

was aligned. After fixing of the brackets, the area around

standards of workmanship expected to be achieved,

the bracket hole was sealed with a special waterproofing

demanded a dynamic construction management with a

compound (see Fig. 11).

high degree of innovativeness, team spirit and quality

Fig II . Marble ftx in g deta ils                             consciousness on the part of staff and workmen.

Anticipating problems in advance and solving them

through trials and mockups was an essential part of site

planning. Further, great emphasis was laid on the

completion of the project within the stipulated time and

cost. Resources were planned and physical progress

1.Stainless steel bracket             monitored through constant review of PERT/CPM

2.Stainl ess steel anchor fastene r

3.Waterp roof resin                   networks.

4.Marble panel

5.Mo ulded rubber cordon with

silico n sealant

6. Sili con sea la nt

7. 8 to IO mm joints between pane ls

8. Concrete shell

9. Curved surface

The alignment of the panels was adjusted at each

layer so that the surface geometry and pattern lines were

maintained. The pieces near edge, re-entrant and cusp

lines were cut to suit the boundary lines. Gaps 8 to 10

mm wide at the joints were filled with moulded rubber

cordon, and the top of the joints, as also the holes in the

marble, sealed with silicon sealant. The entire marble

surface was, lastly, washed with a solution of 30%

muriartic acid mixed in water, to remove dirt and stains.

A specially designed structural steel framework was

provided to accommodate access and working platforms.

The platforms were free from the surface of the shells so

a. house of worship

h. ancillary building

c. public utilities

d. parking

e. main gate

a.- i. pool

j. outer podium

k. bridges

1. entrance

nl. inner haJJ

Ullder COllstructioll

Oh lotus in the heart!

Growing up from the soil

Of mother India.

Drawing deep springs

Up from the depths of Asia,

Rising a mighty fountain

Of mystic power unseen

Felt, almost heard,

As it overflows

From petals clasped in prayer

To carry the voices

Of the singers praising God

To be scattered far and wide

By the scattering angels-

Armfuls of prayer they carry

Like pan nie rs of invisible flowers

Scattering the Words of God

Scattering His Glorious Words

Up to the snow clad Himalayas

Down to the lapping edge of t he seas

A rain of perfume

A rain of blessing

It seeps into every crevice

Showers every jungle

Spatters the deserts' sands

Passes above every meadow "

Blows into every cavel

The scattering angels

Rank on rank, tile on file,

Deploying the promise

Of their Lo rd the Alm ighty.

Madame Ru~!yyih RabMni

Thr Shrinr of the Bab, Martyr-Herdld of the Baha'i Faith, on thr slopes of Mount Carmel. Haifa, Israel.

The Shrine of the Bab is one of the holiest places of pHgrimagr for thr followers of the Baha'i religion.

The monumental terraced gardrns surrounding it are commonly known as -Hanging Gardens of Mount

Cannrl-, and were designed by Farihorz Sahba, the architett of the Baha'i HOllsr of Worship in India,

I

L

I

I NTERNATIONAL R ECOGNITION

The B:1 h:i'j lI ou~t of Wo~ h i p in New Delhi. India has hem rccog n i~cd :1' one of t h e maS\(Tpirn.'s of [W(,lH il'lh-

rt.' llI ury art hi lccltm', :md !la .. WOII ma ny award.. ind uding [Il(' following :

•

rir..l lion our award from Ill(' 11I1l'rrailh Fonml on Rrligiouo; An and Arrhiu'Cl urC. Am li;HC or the American Instilut\'

of Arch itl'I'IS. Wa\hi ngto ll. D.C .. in 1987

•

Spl't:i'l l award from the InslilUlion of Slru('lUrai Enginccrs of Ihl' United Kingdom in 1987

•

The P:lUl Wall:rhury Outdoor Light ing Dl'!>ig n Award- Spl'd:11 Ci lalitJI1. from tlw Ill um ina ting Engi nl'{'ring $ocie1Y                  or

Non h Amcrica in 1988

•

Recognition f rolll the Arl1l:riC;1Il CO IKn' tt l n~l i l utc:1:. one of the n lll" ! t'o rH.'r<'iC ~1n1(,lllr{'" of Ihl' worl d in 1990

•

TIll' GlullArt AcadclIlY 2000 award for -promoting till' II nily a nd harmony or p~oplc or all 11:llion", religio n!> and

!toria] Si rala, 10 an eX\('1lI unsurpassC'd by any Olhtr ,Irchil~clural 1II0nUlll l' Il I worldwidt-

— Architectural Blossoming of the Lotus (Used by permission of the curator)